TP-Link CPE610 v2

OpenWrt + WFB-NG RX-Only Deployment Guide

(Ath79 / MIPS 24KC)

1. Purpose of This Document

This document describes the complete procedure to deploy WFB-NG (RX-only) on a TP-Link CPE610 v2 running OpenWrt, including:

- Why IPK installation fails on this device

- Why a custom OpenWrt firmware is mandatory

- How to build OpenWrt with WFB-NG included

- How to flash the firmware

- How to verify WFB-NG installation

- How to run WFB-NG in RX-only mode

This guide is written based on real deployment experience, not theory.

2. Hardware & Software Overview

2.1 Hardware

| Component | Details |
| --- | --- |
| Device | TP-Link CPE610 v2 |
| SoC | Atheros AR9344 |
| CPU Architecture | MIPS 24KC |
| Flash | 8 MB |
| RAM | 64 MB |
| Wi-Fi | Atheros ath9k (5 GHz) |

2.2 Software Stack

| Layer | Version |
| --- | --- |
| OpenWrt | 24.10.4 |
| Target | ath79 / generic |
| Architecture | mips_24kc |
| RootFS | squashfs |
| Kernel | 6.6.x |
| WFB-NG | 25.01 (OpenWrt build) |

3. Intended Network Topology

Drone (WFB-NG TX)

│

│ 5 GHz raw WiFi

▼

┌─────────────────────────────┐

│ TP-Link CPE610 v2            │

│ OpenWrt + WFB-NG             │

│ RX ONLY (monitor mode)       │

│                              │

│ wfb_rx -f                    │

└─────────────┬───────────────┘

│ Ethernet

▼

┌─────────────────────────────┐

│ Relay Station (Linux / RPi) │

│ wfb_rx -a (Aggregator)      │

└─────────────────────────────┘

Key design decision:

- CPE610 is RX-only

- No TX, no cluster, no overlay installs

- Ethernet is used to forward packets to the relay

4. Why IPK Installation Fails on CPE610

4.1 Overlay Size Limitation

/overlay size: ~1.3 MB

Available: ~450 KB

WFB-NG dependencies:

| Package | Size |
| --- | --- |
| libstdc++ | ~2.0 MB |
| libsodium | ~200 KB |
| wfb-ng | ~500 KB |

➡ Overlay cannot fit required libraries

4.2 Observed Error (Expected)

verify_pkg_installable: Only have 480kb available on filesystem /overlay

pkg libstdcpp6 needs 2070

4.3 Conclusion

❌ Removing packages does NOT help
❌ Overlay cannot be resized on squashfs
❌ External storage is not supported reliably

✅ Correct solution: Build OpenWrt firmware with WFB-NG baked in

5. OpenWrt Image Types Explained

5.1 Factory Image

openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-factory.bin

Used only when:

- Device is running TP-Link stock firmware

- First OpenWrt installation

5.2 Sysupgrade Image

openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-sysupgrade.bin

Used when:

- OpenWrt is already installed

- Upgrading to custom firmware (WFB-NG included)

6. Downloading Official OpenWrt Images

6.1 Download Location

https://downloads.openwrt.org/releases/24.10.4/targets/ath79/generic/

6.2 Download Commands

mkdir -p ~/owrt/downloads

cd ~/owrt/downloads

wget https://downloads.openwrt.org/releases/24.10.4/targets/ath79/generic/openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-factory.bin

wget https://downloads.openwrt.org/releases/24.10.4/targets/ath79/generic/openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-sysupgrade.bin

7. Building WFB-NG IPKs (SDK Method)

7.1 Download OpenWrt SDK

wget https://downloads.openwrt.org/releases/24.10.4/targets/ath79/generic/openwrt-sdk-24.10.4-ath79-generic_gcc-13.3.0_musl.Linux-x86_64.tar.zst

tar --zstd -xf openwrt-sdk-*.tar.zst

cd openwrt-sdk-24.10.4-ath79-generic_*

7.2 Add WFB-NG Package Source

git clone https://github.com/svpcom/wfb-ng.git

cp -a wfb-ng/openwrt/net/wfb-ng package/

cp -a wfb-ng/openwrt/net/wfb-ng-full package/

7.3 Build WFB-NG Packages

make defconfig

make package/wfb-ng/compile V=s

Result:

bin/packages/mips_24kc/base/

├── wfb-ng_*.ipk

└── wfb-ng-tun_*.ipk

⚠ These IPKs cannot be installed on CPE610
They are used only to embed into firmware

8. Building Custom Firmware with ImageBuilder

8.1 Download ImageBuilder

wget https://downloads.openwrt.org/releases/24.10.4/targets/ath79/generic/openwrt-imagebuilder-24.10.4-ath79-generic.Linux-x86_64.tar.zst

tar --zstd -xf openwrt-imagebuilder-*.tar.zst

cd openwrt-imagebuilder-24.10.4-ath79-generic.Linux-x86_64

8.2 Add WFB-NG Packages

mkdir -p package/network/utils

cp -a ~/owrt/wfb-ng/openwrt/net/wfb-ng package/network/utils/

8.3 Build Firmware for CPE610 v2

make image PROFILE=tplink_cpe610-v2 \

PACKAGES="wfb-ng iw ca-bundle -luci -uhttpd -uhttpd-mod-ubus"

8.4 Output Files

bin/targets/ath79/generic/

├── openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-factory.bin

└── openwrt-24.10.4-ath79-generic-tplink_cpe610-v2-squashfs-sysupgrade.bin

9. Flashing the Custom Firmware

9.1 Backup Current Config (Optional)

sysupgrade -b /tmp/backup.tar.gz

9.2 Copy Firmware

scp openwrt-*-sysupgrade.bin root@<CPE_IP>:/tmp/

9.3 Flash

sysupgrade /tmp/openwrt-*-sysupgrade.bin

⚠ SSH will disconnect during upgrade
⚠ Wait 2–3 minutes for reboot

10. Verifying WFB-NG Installation

10.1 Check Binaries

which wfb_rx

which wfb_tx

Expected:

/usr/bin/wfb_rx

/usr/bin/wfb_tx

10.2 Check Version

wfb_rx --help

Expected:

WFB-ng version 25.01-1-openwrt

10.3 Check Dependencies

ldd /usr/bin/wfb_rx

10.4 Check Installed Packages

opkg list-installed | grep wfb

Expected:

wfb-ng

wfb-ng-tun

WFB-NG Distributed Setup (GS = vind-rly Ubuntu, Node = CPE610 OpenWrt) — Full Procedure

0) Topology and fixed values (your setup)

- GS / Center node: vind-rly (Ubuntu/RPi5)
IP: 10.5.6.100

- CPE / Node: CPE610 (OpenWrt)
IP: 10.5.6.102

- Client/Laptop (example): 10.5.6.50

- RF channel: 157 HT20

- Monitor interface on CPE: phy0-mon0

- WFB Link ID: 7669206

- Important constraint: CPE is single radio → cannot keep STA Wi-Fi and WFB monitor reliably at the same time. When WFB runs, STA must be stopped.

3) CPE side (OpenWrt) — install + configure WFB RX forwarding

3.1 Confirm WFB tools exist on OpenWrt

On CPE:

opkg list-installed | grep -i wfb

ls -l /usr/bin/wfb_*

Expected: wfb_rx, wfb_tx exist.
(Usually no wfb-server on OpenWrt → that’s normal.)

OpenWrt CPE node (10.5.7.102) — monitor interface setup for cluster-ssh

A1) Disable netifd wireless management (important)

We do this so OpenWrt doesn’t delete/recreate interfaces and break phy0-mon0.

uci set wireless.radio0.disabled='1'

# if you have wifi-iface sections, disable them too (example name may differ):

# uci set wireless.default_radio0.disabled='1'

uci commit wireless

wifi down

Confirm wifi is down:

wifi status

A2) Create the radio init script used by the center node

Create: /usr/sbin/wfb-mon0.sh

cat > /usr/sbin/wfb-mon0.sh <<'EOF'

#!/bin/sh

set -e

# ---- Set your RF settings here ----

REG="IN"            # regulatory domain (optional)

CHAN="157"          # channel number (example: 157 = 5785 MHz)

HT="HT20"           # HT20 / HT40+ / HT40-

# ---- Apply reg domain (best effort) ----

iw reg set "$REG" 2>/dev/null || true

# ---- Recreate monitor interface cleanly ----

iw dev phy0-mon0 del 2>/dev/null || true

iw phy phy0 interface add phy0-mon0 type monitor flags otherbss || true

ip link set phy0-mon0 up

iw dev phy0-mon0 set monitor otherbss 2>/dev/null || true

# ---- Set RF channel ----

iw dev phy0-mon0 set channel "$CHAN" "$HT"

EOF

chmod +x /usr/sbin/wfb-mon0.sh

Test it (must succeed)

/usr/sbin/wfb-mon0.sh

ip link show phy0-mon0

iw dev phy0-mon0 info

You should see type monitor and your channel.



Make wfb-mon0.sh a proper OpenWrt service

cat > /etc/init.d/wfb-mon0 <<'EOF'

#!/bin/sh /etc/rc.common

START=25

STOP=10

start() {

/bin/sh /usr/sbin/wfb-mon0.sh

}

stop() {

ip link del phy0-mon0 2>/dev/null || true

}

status() {

if iw dev 2>/dev/null | grep -q "Interface phy0-mon0"; then

echo "OK: phy0-mon0 exists"

iw dev phy0-mon0 info 2>/dev/null | sed 's/^/  /'

return 0

else

echo "NOT OK: phy0-mon0 missing"

return 1

fi

}

EOF

chmod +x /etc/init.d/wfb-mon0



/etc/init.d/wfb-mon0 enable

/etc/init.d/wfb-mon0 start

ls -l /etc/rc.d/ | grep wfb-mon0

logread -e wfb-mon0 | tail -n 50

8) CPE SSH authentication tweaks

You needed cluster ssh to run remote commands on OpenWrt and hit:

- ash: /bin/bash: not found

8.1 Fix /bin/bash missing (OpenWrt)

On CPE:

ln -s /bin/ash /bin/bash 2>/dev/null || true

/bin/bash -c 'echo bash_shim_ok'

8.2 SSH key-based login (GS → CPE)

On GS, create key if not already:

sudo mkdir -p /root/.ssh

sudo ssh-keygen -t ed25519 -f /root/.ssh/wfb_cluster_ed25519 -N ""

Copy public key to CPE (manual method):

sudo ssh-copy-id -i /root/.ssh/wfb_cluster_ed25519.pub root@10.5.6.102

If ssh-copy-id is not available, do manual:

sudo cat /root/.ssh/wfb_cluster_ed25519.pub | ssh root@10.5.6.102 "cat >> /etc/dropbear/authorized_keys"

Now test:

sudo ssh -i /root/.ssh/wfb_cluster_ed25519 root@10.5.6.102 'echo ok'

B) Center node / Relay GS (10.5.7.100) — local node + cluster keys

We will run GS as cluster ssh with two nodes:

- 127.0.0.1 → local USB adapter (wlx00c0cab6db3b)

- 10.5.7.102 → CPE monitor (phy0-mon0) created by script

B1) Generate the cluster SSH key (one-time)

This is the key the center node uses to SSH into nodes.

sudo -i

ssh-keygen -t ed25519 -f /root/.ssh/wfb_cluster_ed25519 -N "" -C "wfb-cluster"

chmod 700 /root/.ssh

chmod 600 /root/.ssh/wfb_cluster_ed25519

Allow this same key for root@localhost (local node)

cat /root/.ssh/wfb_cluster_ed25519.pub >> /root/.ssh/authorized_keys

chmod 600 /root/.ssh/authorized_keys

ssh -i /root/.ssh/wfb_cluster_ed25519 root@127.0.0.1 'echo LOCAL_NODE_OK'

You must see: LOCAL_NODE_OK

B2) Copy the public key to the CPE (node) for passwordless SSH

From center node:

sudo -i

ssh-copy-id -i /root/.ssh/wfb_cluster_ed25519.pub root@10.5.7.102

ssh -i /root/.ssh/wfb_cluster_ed25519 root@10.5.7.102 'echo CPE_NODE_OK'

You must see: CPE_NODE_OK

C) Update /etc/wifibroadcast.cfg (center node) — cluster with local node + CPE node

Edit:

sudo nano /etc/wifibroadcast.cfg

Use this cluster block (keep it exactly like this style):

[cluster]

nodes = {

'127.0.0.1':  { 'wlans': ['wlx00c0cab6db3b'] },

'10.5.7.102': { 'wlans': ['phy0-mon0'], 'custom_init_script': '/usr/sbin/wfb-mon0.sh' }

}

ssh_user = 'root'

ssh_port = 22

ssh_key = '/root/.ssh/wfb_cluster_ed25519'

server_address = '10.5.7.100'

base_port_server = 10000

base_port_node = 11000

api_port = 8203

stats_port = 8303

Important notes

- Do NOT run --wlans together with --cluster (that’s why you got the CLI error earlier).

- The node “wlans” names must be:

  - local: the real USB iface name on center node (wlx...)

  - CPE: the monitor iface name created by script (phy0-mon0)

9) Run cluster (foreground or background)

9.1 Foreground

sudo wfb-server --profiles gs --cluster ssh